LhARA will be a hybrid accelerator system in which laser interactions drive the creation of a large flux of protons or light ions that are captured using a plasma (Gabor) lens and formed into a beam to deliver Particle Beam Therapy. Preclinically this can be used to radio-biological interactions while clinical development will allow treatment of cancer patients

Proposed development

LhARA will be a hybrid accelerator system in which laser interactions drive the creation of a large flux of protons or light ions that are captured using a plasma (Gabor) lens and formed into a beam. This will allow the delivery of particle-beam therapy to be delivered.

The laser-driven source allows protons and ions to be captured at energies significantly above those produced in conventional facilities where cyclotrons produce the particles,. This will overcome the  space-charge limit on the instantaneous dose rate that can be delivered.

Problem addressed

Traditional Radiotherapy uses high energy X-ray (photons) to treat tumours. Particle beam therapy . offers advantages by depositing energy more precisely within tumours, sparing healthy tissue. However, existing proton machines are large and expensive and do not offer a range of particles or energies.

How LhARA Addresses It: Proton beam machines are mainly cyclotron based and are often expensive to use and operate, so there exists a need to develop smaller, cheaper and more flexible machines. LhARA aims to develop smaller, more flexible machines for producing a range particles at different energies by using high-power lasers. These machines will be able to deliver short, intense pulses and micro-beams, enhancing cancer cell killing while minimizing harm to healthy tissue. LhARA aims to open Particle Beam Therapy (PBT) to the many and enable the investigation of radio-biological interactions to improve treatments and explore laser-hybrid accelerators.

Technology overview

LhARA will integrate three cutting-edge technologies:

  - Laser-Driven Proton & Ion Source: This component generates short, intense pulses for “FLASH” radiation and tightly focused mini-beams. Unlike traditional collimation methods, LhARA achieves this without collimation.

  - Electron Plasma (Gabor) Lens: Laser-driven ion sources create highly divergent beams, with a large energy spread and an energy that can vary up to 25% pulse by pulse. A Gabor lens is a cost-effective alternative to conventional solenoids, the Gabor lens provides strong focusing capabilities

  - Post Acceleration using Fixed-Field Alternating (FFA) Gradient Accelerator: Rapid acceleration will be performed using a Fixed-field alternating-gradient accelerator which allows flexibility in adjusting the time, energy, and spatial structure of the ion beam. Collaboration with major UK institute groups in ion-source lasers and accelerators ensures robust development. Relevant technologies include software/simulation, instrumentation, automation, and simulation packages.

These technologies will have wide reaching applications for a range of other …

A paper describing the technology can be found here: https://doi.org/10.3389/fphy.2020.567738